Pre-cruise

- Slacken starboard stern line (on port transom cleat) to allow for easy boarding (i.e., within 6-12” of dock)

- Open doors, windows, and blinds as desired

- Roll up flybridge canvas

  - Forward windscreen

  - Side curtains

  - Aft curtain

  - Aft door curtain

- Remove instrument covers (flybridge and lower helm)

- Inspect engine room

  - Oil level (lower starboard side)

  - Coolant level (upper port side, forward)

  - Bilges

  - Seacocks (all open)

Engine Start

- DC panel, energize:

  - DC MAIN

  - ENG #1

- Listen for throttle beep

- Press and hold button on throttle until beeping stops and red light on quadrant is steady

- Press ENGINE START switch

- Check RPM, eng temp, oil press steady

- DC panel, energize:

  - BRIDGE MAIN

  - REFRIGERATOR

  - FRESH WATER

  - HEAD

- Flybridge, energize:

  - AUTOPILOT POWER

  - ACC

  - Chartplotter

  - VHF

Generator Start

- DC panel, push generator START/STOP switch to right for 1-2 seconds

- Allow generator to warm up for 3-5 minutes

- Disconnect shore power

  - Unplug from dock

  - Unplug from boat

  - Lay cord on dock

- Energize AC panel

  - SHORE POWER switches OFF (down)

  - Sliders to cover SHORE POWER
switches

- AC panel, energize:

  - REFRIGERATOR

  - OUTLETS #1

  - OUTLETS #2

  - WATER HEATER

  - BATT CHARGER

  - Other accessories as needed

GENERATOR switch ON (up)

Air Conditioning/Heating

- AC panel, energize:

  - AIR COND. 1

  - AIR COND. 2

  - A/C PUMP

- Ensure water flow out of port
through-hulls (aft and forward)

- Adjust temperature on thermostats 
as needed (one at salon entrance and
one in stateroom, port bulkhead on 
closet)

Departure

- Ensure chartplotter, VHF, autopilot on (upper and lower helms)

- Cast off initial lines (all lines remain on dock)

  - Bow lines

  - Forward spring

  - Starboard stern (port transom cleat)

- Cast off final lines

  - Aft spring

  - Port stern (starboard transom cleat)

- Keep boat off dock and outboard piling by hand as able

- Idle reverse with bow thruster

  - Remain aligned with slip (not too close to docked boats on right)

  - When well clear of last docked boat:

    - LEFT bow thruster

    - FULL LEFT rudder

    - FWD throttle 1000 RPM

Anchoring

- If at lower helm, transfer command to flybridge

- Bring boat to full stop into wind (less than 1 KT on GPS)

- Energize WINDLASS switch on left side of panel

- Open anchor locker and remove
pin from anchor chain

- Operate anchor up/down using foot
pedals next to anchor locker or
switch on right side of flybridge
panel

- Note: anchor may need nudge to initially drop

- Set sufficient scope and let out additional slack line to allow for securing

- Secure anchor line to Sampson post (do not allow tension directly on windlass)

- To raise anchor:

  - Start engine

  - Remove line from Sampson post

  - Ensure light tension on line (slack line will jam in windlass)

  - Use foot pedals to raise, monitoring closely for jam

  - Maneuver boat to reduce load on windlass

  - If jam occurs, reverse line to undo jam

  - Help anchor over roller if needed and secure pin in chain

Returning

- Drive from lower helm only while docking

- Align with slip as soon as entering cove

- Slow to near stop when within 1 boat length of slip

- Remain aligned with rudder and thruster

  - Before applying forward nudge in close, ensure at a complete stop or slight reverse

- Grab port stern line, starboard stern line, and set both loose in cockpit

- Grab aft spring line and secure at mark

- Secure stern lines

- Secure bow lines

- Shut down all electronics

  - Lower helm

    - Chartplotter

  - Upper helm

    - Chartplotter

    - VHF

    - Autopilot

    - Accessory switch to OFF

- Engine OFF

Generator Shutdown

- AC panel, switch off all except:

  - REFRIGERATOR

  - BATT. CHARGER

- GENERATOR SWITCH off (down)

- Allow generator to cool down 2-3 minutes

- DC panel, push generator START/STOP switch
to left to shut off generator

Performance Table (Ideal)

Water System Sanitization

- Prepare a chlorine solution using one gallon of water and 1/4 cup household bleach (5% sodium Hypochlorite solution ). Use one gallon of solution for each 5 gallons of tank capacity. With tank empty, pour chlorine solution into tank. Given Paikea’s 70 gallon fresh water tank, 3.5 cups of bleach should be poured into full water tank.

- Complete filling of tank with fresh water. Open each faucet and drain cock until air has been released and the entire system is filled. Do not turn off the pump; it must remain on to keep the system pressurized and the solution in the lines

- Allow to stand for at least three hours, but no longer than 24 hours.

- Drain through every faucet on the boat (and if you haven't done this in a while, it's a good idea to remove any diffusion screens from the faucets, because what's likely to come out will clog them).

- Fill the tank again with fresh water only, drain again through every faucet on the boat.

- To remove excess chlorine taste or odor which might remain, prepare a solution of one quart white vinegar to five gallons water and allow this solution to agitate in tank for several days by vessel motion.

- Drain tank again through every faucet, and flush the lines again by filing the tank 1/4-1/2 full and again flushing with potable water.