Automatic Switch for Small Refrigerator

Parts:

Radio Shack 125vac DPDT 10amp Relay, part# 275-217. This relay is rated for

10 amps, about 1200 watts, so the refrigerator shouldn’t draw more  amperage than a small refrigerator. DO NOT confuse this relay with the part# 275-218, which is a 12vdc component!!

Male electrical plugs for AC—need 2

Female electrical plug for AC---need 1

Wire-16 gauge flexible

Wiring:

See the “schematic” below for the wiring.  I wired the relay so that the default

power source would be the inverter. In other words, if there is no power from the shore/generator source, power from the inverter is used without

activating the relay coil.  The numbers, 1-8, represent the posts on the bottom of the relay.

I didn’t use the ground wire from the shore/generator power source.  This may have been a mistake and I plan to run a ground wire directly from the OEM shore/generator outlet next to the refrigerator to the three wire female plug that the refrigerator power cord plugs into.

The female socket is connected to the “refrigerator wire”.  The 2 male plugs are connected to the inverter and shore/generator power cords.

Make sure that the polarization of the AC from the inverter and shore/generator power sources is not reversed.  I ran the wire from the left side of the female socket/outlet (facing the outlet) on the inverter to post number 1 on the relay.  I ran the wire from the left side of the socket/outlet (facing the outlet) from the Norcold’s shore/generator power source to post number 7 on the relay.  Then the wire from post number 5 must connected to the left side (facing the female socket) which is attached to the “refrigerator wire”. Then the refrigerator power cord is plugged into the socket on the end of the “refrigerator wire”.

I used an adhesive to attach the relay to the inside of a plastic outdoor electric box.  I put this box next to the OEM AC outlet that the Norcold had been plugged into. After 2 days of shorepower, the relay wasn’t warm.